Section 507.03 of the Standard Specifications for Highway Construction, Edition of 2003, is hereby amended as follows:

The first paragraph of Subsection 507.03 (a) is hereby deleted and the following substituted therefore:

(a) Installation of Dowel Bars and Tie Bars.  Dowel bars and tie bars shall be installed according to details included on the plans.  Drills used to make holes shall be held in a rigid frame to assure proper horizontal and vertical alignment.  The equipment shall be operated so as to prevent damage to the pavement being drilled.  The drilling procedure shall be approved by the Engineer.  When using a resin anchoring system, the bars shall be installed according to information provided by the manufacturer.  To secure the bars, special care must be taken to ensure that the filling system completely surrounds the bars and fills the holes.