Sections 604, 617, 731, 732 and 734 of the Standard Specifications for Highway Construction, Edition of 2003, are hereby amended as follows:

The first paragraph of Subsection 604.02(a) is deleted and the following is substituted therefore:

All work zone traffic control devices used on the project, including sign supports, barricades, traffic drums equipped with flashing lights, crash cushions, and impact attenuators shall comply with the requirements of National Cooperative Highway Research Program (NCHRP) Report 350 or the Manual for Assessing Safety Hardware (MASH).  The Contractor shall furnish a certification of such compliance from the manufacturer or supplier of all work zone traffic control devices prior to using the devices on the project.  The certification shall state the device meets the requirements of NCHRP 350 or MASH and include a copy of the Federal Highway Administration’s (FHWA) approval letter with all attachments for each device.  Devices shall be fabricated and installed in accordance with the plans and with the crash testing documentation provided in the FHWA approval letter, which is available at http://safety.fhwa.dot.gov/roadway_dept/policy_guide/road_hardware/.  The 2lb. (0.9 kg) minimum channel post or 4” x 4” (100 mm x 100 mm) wood post sign support systems, installed in accordance with the plans (direct buried), have been previously tested and accepted, and, therefore, do not require certification.  No direct payment will be made for fulfilling the requirements of this Specification, but full compensation will be considered included in the contract unit prices bid for the various traffic control devices.

The third sentence of paragraph three of Subsection 617.01 is deleted and the following sentence substituted therefore:

The guardrail terminal shall satisfy the National Cooperative Highway Research Program (NCHRP) Report 350 or the Manual for Assessing Safety Hardware (MASH) for a test level 3 (TL-3) terminal.

The first sentence of paragraph one of Subsection 617.02(f) is deleted and the following sentence substituted therefore:

The Contractor shall furnish a certification from the manufacturer or supplier that the guardrail terminal meets the requirements of NCHRP Report 350 or MASH for a TL-3 terminal.

The second sentence of Subsection 731.01 is deleted and the following sentence substituted therefore:

The attenuation barrier shall satisfy the National Cooperative Highway Research Program (NCHRP) Report 350 or the Manual for Assessing Safety Hardware (MASH) requirements for a Test Level 3 (TL-3) crash cushion.

The first sentence of paragraph one of Subsection 731.02 is deleted and the following sentence substituted therefore:

The Contractor shall furnish a certification from the manufacturer or supplier that the impact attenuation barrier meets the requirements of NCHRP Report 350 or MASH for a TL-3 crash cushion.

The second sentence of Subsection 732.01 is deleted and the following sentence substituted therefore:

The crash cushion shall satisfy the National Cooperative Highway Research Program (NCHRP) Report 350 or the Manual for Assessing Safety Hardware (MASH) requirements for a Test Level 3 (TL-3) crash cushion.

The first sentence of Subsection 732.02(b) is deleted and the following sentence substituted therefore:

The Contractor shall furnish a certification from the manufacturer or supplier that the crash cushion meets the requirements of NCHRP Report 350 or MASH for a TL-3 crash cushion.

The second sentence of Subsection 734.01 is deleted and the following sentence substituted therefore:

The bridge end terminal shall satisfy the National Cooperative Highway Research Program (NCHRP) Report 350 or the Manual for Assessing Safety Hardware (MASH) requirements for a Test Level 3 (TL-3) terminal.

The first sentence of Subsection 734.02(b) is deleted and the following sentence substituted therefore:

The Contractor shall furnish a certification from the manufacturer or supplier that the bridge end terminal meets the requirements of NCHRP Report 350 or MASH for a TL-3 terminal.