Division 600 of the Standard Specifications for Highway Construction, Edition of 2003, is hereby amended as follows:

section 620, SEEDING, is amended as follows:

Paragraphs (1) and (2) of Subsection 620.03(f), Water, are hereby deleted and the following substituted therefore:

(f) Water.  (1) Initial Application.  From April 1 through December 31, either the day before the seeding is placed or on the day of the seeding operation (either before the seed is placed or after the application of the mulch cover), a minimum of 20.4 M Gallons per acre (188 cu m or 188 kL per ha) of water will be applied to thoroughly moisten the soil to the depth of pulverization and then as necessary to germinate the seed.  This quantity may be reduced by the Engineer dependent on the soil moisture conditions immediately prior to the application of the seeding.  Failure to apply the initial application of the quantity of water directed by the Engineer will result in a deduction in payment as shown below.  Water used for hydro-seeding or tackifier application will not be measured or paid for, and will not be included in the quantity of water required for the initial water application.  The initial application of water and deductions for failure to water will not be required from January 1 through March 31.

(2) Weekly Application.  From April 1 – December 31, unless otherwise directed by the Engineer, the Contractor shall apply water in an amount such that, in conjunction with any rainfall, the seeded and mulched areas will receive an amount equivalent to a minimum of ¾” (19 mm) of water each week beginning the week after seeding and continuing for a minimum of four (4) weeks (¾” [19 mm] of water is equivalent to 20.4 M Gallons per acre [ 188 cu m or 188  kL per ha]).  The Engineer will adjust the amount of water required each week to deduct any rainfall received during the 7 calendar day period prior to the weekly watering.  The weekly applications of water and deductions for failure to water will not be required from January 1 through March 31.

section 621, temporary erosion control items and devices, is amended as follows:

The fourth and fifth paragraphs of Subsection 621.03(b), Temporary Seeding, are hereby deleted and the following substituted therefore:

From April 1 through December 31, either the day before the temporary seeding is placed or on the day of the temporary seeding operation (either before the seed is placed or after the application of the mulch cover) an application of water will be applied, in accordance with Subsection 620.03(f)(1).  No subsequent weekly waterings will be required for Temporary Seeding.

Failure to meet this application of water requirement will result in a permanent deduction in payment and /or permanent recovery of payments equal to the minimum bid price established in Subsection 620.05(d) for each M.G. (kL) not applied as directed in accordance with these specifications.  Equipment and methods used to place the water shall be in accordance with Subsection 620.03(f)(4).

section 624, SOLID SODDING, is amended as follows:

The second paragraph of Subsection 624.03(c), Placement of Sod, is hereby deleted and the following substituted therefore:

When sodding is completed, the sodded areas shall be cleared of loose sod, excess soil, or other foreign material; a thin application of topsoil shall be scattered over the sod as a top dressing; and the areas thoroughly moistened.  Water shall be applied at a minimum rate of 20.4 M Gallons per acre (188 cu m or 188 kL per ha) or as directed by the Engineer for a period of at least 3 weeks.  The Engineer will adjust the amount of water required each week to deduct any rainfall received during the 7 calendar day period prior to the weekly watering.  The weekly applications of water and deductions for failure to water will not be required from January 1 through March 31.  The time required for application of water will not be included in the computation of contract time for completion of the project provided all other work under the Contract has been completed.

Failure to meet this water application requirement will result in a permanent deduction in payment and /or permanent recovery of payments equal to the minimum bid price established in Subsection 620.05(d) for each M.G. (kL) not applied as directed in accordance with these specifications.  Equipment and methods used to place the water shall be in accordance with Subsection 620.03(f)(4).