Section 501 of the Standard Specifications for Highway Construction, Edition of 2003, is hereby amended as follows:

- Subsection 501.02 (g)(3) is hereby deleted and the following substituted therefore:

501.02 (g)(3) Tie Bars. No coating repairs will be required for uncoated cut or sheared ends of tie bars. Tie bars that must be bent and later straightened to facilitate construction shall comply with AASHTO M 31, Grade 40 (Grade 300). Tie bars shall be bent back reasonably straight and any damage to the epoxy coating during the rebending shall be repaired in accordance with (1) above. Tie bars broken during the rebending shall be replaced by the Contractor at no cost to the Department.  Installation of replacement tie bars shall use an approved non-shrink grout or a resin anchoring system listed on the Department’s Qualified Products List. The diameter of the drilled holes and the installation procedures shall be as recommended by the grout manufacturer or the resin anchoring system manufacturer.