Section 804 of the Standard Specifications for Highway Construction, Edition of 2003, is hereby amended as follows:

The last paragraph of Section 804.06 is hereby deleted and the following substituted therefore:

804.06 Placing and Fastening.  Reinforcing steel that is to be doweled into existing concrete shall be installed into drilled holes and secured using an approved non-shrink grout or a resin anchoring system listed on the Department’s Qualified Products List. The diameter of the drilled holes and the installation procedures shall be as recommended by the grout manufacturer or the resin anchoring system manufacturer.